OFDM Joint Communications and Radar (JCAS)

Technical Documentation

Repository: smanthat/OFDM  |  June 2026

Status: Communications path validated on hardware (BER ~0.02 on AD2 loopback).

This document is the developer reference for the system as implemented. It is written so that any future work (radar path, CFO correction, FPGA port) can build directly on it without re-deriving decisions. Read alongside the code; the code is canonical where they disagree.

# 1. System Overview

An OFDM physical layer designed from the start to serve two functions with one waveform:

- Communications: QPSK data over 64 subcarriers, blind frame synchronization, channel estimation, and equalization. Implemented and hardware-validated.

- Radar: range-Doppler imaging using the same frame via symbol division and 2D FFT. Designed, not yet implemented. See Section 8 (Extension Points).

End goal: real-time implementation in hand-written SystemVerilog on a Nexys A7-100T (Artix-7 xc7a100tcsg324-1), with the FFT shared between the comms and radar paths.

## 1.1 Signal Flow (current, comms path)

bits.txt (2048 bits)

-> QPSK_Mapper -> Symbol_Mapper -> [prepend 2 preambles]

-> IFFT per block -> add CP (16 samples) -> flatten -> normalize -> (quantize)

-> ad2_i_waveform.csv / ad2_q_waveform.csv

-> [AD2 AWG W1/W2, SYNCHRONIZED, Run All]

-> [physical loopback / channel]

-> [AD2 scope Ch1=I, Ch2=Q capture] -> Recieved_waveform.csv

-> Schmidl-Cox sync on RAW 1-D stream -> d_peak

-> slice 18 blocks from d_peak -> strip CP -> FFT per block

-> H_est = freq[1] / channel_preamble   (full-spectrum preamble)

-> data_eq = freq[2:] / H_est           (zero-forcing equalize)

-> Demapper -> BER vs bits.txt

# 2. Physical Layer Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Subcarriers N | 64 | FFT size |
| Bandwidth | 500 kHz | Limited by AD2 AWG practical rate |
| Sample rate | 500 kS/s | Complex baseband |
| Subcarrier spacing | 7.8125 kHz | BANDWIDTH / N |
| OFDM symbol duration | 128 us | N / SAMPLE_RATE |
| CP length CP_LEN | 16 samples (32 us) | Hardcoded, Formula: int(np.ceil(T_GI * BANDWIDTH)) Must exceed channel delay spread; sim channel max tap = 7 |
| Block length BLOCK_LEN | 80 samples | N + CP_LEN |
| Modulation | QPSK, Gray-coded | 2 bits per symbol |
| Frame | 18 blocks | Hardcoded, 1 timing preamble + 1 channel preamble + 16 data |
| Samples per frame | 1440 | 18 x 80 |
| Payload | 2048 bits | Exactly 16 x 64 x 2 - must divide evenly (Section 7.5) |
| Fixed-point budget | W=16, F=13 | From BER-vs-bit-width sweep (Section 6) |

Invariant: payload bits = 2 x N x NUM_DATA_SYMBOLS. Any other length silently desynchronizes the frame (bug 7.5).

# 3. Frame Structure

block index :    0          1            2 ... 17

contents    : [TIMING]  [CHANNEL]    [DATA x 16]

(SC sync)  (H_est)

Each block on the wire: [CP: last 16 samples of symbol][64-sample IFFT output].

## 3.1 Timing Preamble (block 0)

- Pseudo-random QPSK on even subcarriers only; odd subcarriers zero.

- Even-only spectrum produces a time-domain signal with two identical halves: x[n] = x[n + N/2]. This is the property Schmidl-Cox detects.

- PN (not constant) values spread energy over all 64 samples. A constant value (e.g. 1+1j on every even bin) produces an impulse train - roughly 97% of samples near zero - which destroys the sync metric and stresses the DAC (bug 7.3).

- Even bins scaled by sqrt(2) to restore full symbol power (half the bins are off; power scales as amplitude squared, so amplitude x sqrt(2) doubles the power).

## 3.2 Channel Preamble (block 1)

- Pseudo-random QPSK on all 64 subcarriers - no zero bins, so the receiver division H_est = freq[1] / channel_preamble is defined on every subcarrier (bug 7.4).

- Uses a different seed from the timing preamble so the two are statistically independent (prevents the timing correlator from partially locking onto block 1).

## 3.3 Seeds

Defined once in preamble_gen.py and imported by both transmitter and receiver:

SC_SEED = 0xB00B3E5     # timing preamble

CH_SEED = 0xDEADBEEF    # channel preamble

If TX and RX ever generate preambles from separate code paths, they will eventually disagree. Keep generation in one shared module - this is a hard rule.

# 4. Module Reference

## 4.1 preamble_gen.py

Shared preamble generation. Imported by TX and RX.

timing_preamble_gen(N)  -> complex ndarray (N,)

# PN-QPSK on even bins x sqrt(2), zeros on odd bins.

# Property: ifft(out)[:N//2] == ifft(out)[N//2:]

channel_preamble_gen(N) -> complex ndarray (N,)

# PN-QPSK on all bins. No zeros.

_QPSK_MAP = [1+1j, 1-1j, -1+1j, -1-1j] is internal to preambles only; it is NOT the same ordering as the data mapper and does not need to be (preambles only need TX/RX self-consistency, not Gray-code structure).

## 4.2 QPSK_Mapper.py / Demapper.py

Data-path bit-to-symbol conversion. Gray-coded, quadrant-based:

| Bits | Symbol | Quadrant |
| --- | --- | --- |
| 00 | +1+1j | I >= 0, Q >= 0 |
| 01 | -1+1j | I < 0, Q >= 0 |
| 11 | -1-1j | I < 0, Q < 0 |
| 10 | +1-1j | I >= 0, Q < 0 |

These two files are a matched pair - round-trip verified exact (BER 0.0 on perfect symbols). Do not change one without the other. The demapper makes hard decisions by sign of real/imag, which is equivalent to nearest-constellation-point for QPSK; no separate snapping step is needed before it (Section 7.7).

demapper(signals, original_len, original_len_symbol) truncates output to original_len bits, which removes pad bits added by symbol_mapper.

## 4.3 Symbol_Mapper.py

symbol_mapper(bits, N) reshapes a 1-D complex symbol stream into (num_symbols, N) rows, zero-padding the tail to a multiple of N. Returns (N_symbols, original_len_symbol). The pad symbols are zeros - they decode to garbage bits, which is why the demapper truncation to original_len matters.

## 4.4 transmitter_main.py

Top-level TX. Key sections in order:

- Parameters (Section 2). NUM_BLOCKS = 18 is currently hardcoded - recommended fix is NUM_BLOCKS = 2 + N_symbols.shape[0] so a wrong-length bits.txt cannot silently desync the frame (bug 7.5).

- Bits -> QPSK -> (16, 64) data matrix.

- vstack(timing_preamble, channel_preamble, data) -> (18, 64).

- np.fft.ifft(..., axis=1) -> time domain per block.

- CP prepend: concatenate((block[:, -CP_LEN:], block), axis=1) -> (18, 80).

- Flatten -> scale to 0.8 x max for AD2 voltage range.

- Optional fixed-point quantization (dec_to_fixed_point, W=16, F=13). Note: the current file has i_wave_fixed = i_wave (quantization bypassed) and F = 10. For the float hardware path that is fine; for fixed-point experiments re-enable the call and set F = 13.

- Write ad2_i_waveform.csv, ad2_q_waveform.csv, and transmitted_bits.txt with the 7-field metadata footer.

orignal_length,original_len_symbol,N,BANDWIDTH,CP_LEN,TS,NUM_BLOCKS

2048,1024,64,500000,16,2e-06,18

The metadata footer is the TX-to-RX contract. The receiver parses fields by index; changing the format requires changing both files.

Self-check (prints True when the SC property holds):

preamble_time = np.fft.ifft(preamble_freq, axis=1)

np.allclose(preamble_time[0, :32], preamble_time[0, 32:])   # row 0, COLUMN halves

Note the indexing: [0, :32] is row 0, first 32 samples. preamble_time[0:32] (a row slice) is meaningless here and was a recurring bug (Section 7.1 family).

## 4.5 receiver_communication.py

Top-level RX. Stages:

### (a) Load capture

Reads Recieved_waveform.csv (WaveForms scope export, #-prefixed header comments). Ch1 -> I, Ch2 -> Q. NaN rows masked. Produces the raw complex stream r.

### (b) Schmidl-Cox sync - on the raw 1-D stream, before any reshape

L = N // 2

num = len(r) - N                    # == len(r) - 2L since N = 2L

for d in range(num):

p   = sum_m  conj(r[d+m]) * r[d+m+L]     # m = 0..L-1

r_c = sum_m  |r[d+m+L]|^2

metric[d] = |p|^2 / (r_c^2 + 1e-12)

- p: when the window sits on the preamble, every term is positive-real and the sum is large; elsewhere terms have random phase and cancel.

- metric is in approximately [0, 1], amplitude-invariant, and CFO-immune for timing.

- The metric plateaus over approximately CP_LEN samples (the CP region also satisfies the half-repeat property); any d_peak on the plateau lands inside the CP, which is why a few samples of timing error are harmless.

Energy gating (rejects false peaks where r_c near 0 makes the ratio blow up):

typical = np.median(energy[energy > 0])

metric[energy < 0.2 * typical] = 0      # ZERO OUT - do not boolean-filter!

d_peak = int(np.argmax(metric))

Zeroing keeps the array full-length so d_peak remains a valid sample index. Boolean filtering (metric = metric[mask]) shrinks the array and breaks the index-to-sample correspondence (bug 7.6).

### (c) Frame slice and FFT

frame = r[d_peak : d_peak + BLOCK_LEN * NUM_BLOCKS]   # NUM_BLOCKS from metadata

assert len(frame) == BLOCK_LEN * NUM_BLOCKS           # short-frame guard

freq  = np.fft.fft(frame.reshape(-1, BLOCK_LEN)[:, CP_LEN:], axis=1)   # (18, 64)

### (d) Channel estimate and equalize

H_est   = freq[1] / channel_preamble_gen(N)    # row 1 = channel preamble

data_eq = freq[2:] / (H_est[None, :] + 1e-12)  # rows 2..17 = data

Row map: freq[0] timing preamble (sync only - never divide by it), freq[1] channel preamble, freq[2:] data. An off-by-one here gives BER 0.5 with no crash.

### (e) Demap and BER

data_eq.flatten()[:original_len_symbol] -> demapper -> compare to bits.txt.

Healthy run signature (hardware): metric_peak >= 0.7, median ~0.02 (at least 25x ratio), non-finite H_est bins = 0, BER ~0.02.

## 4.6 Multipath_channel.py

Simulation channel for offline validation: 3 taps h[0]=1.0, h[3]=0.5, h[7]=0.2 plus AWGN at configurable SNR. Max delay 7 samples means CP_LEN = 16 is sufficient. Returns (rx_signal, H, h).

## 4.7 fixed_point_reference.py

dec_to_fixed_point(x, F, W): scale by 2^F, np.round, saturate to [-2^(W-1), 2^(W-1)-1], return ints. Requirements baked in from earlier debugging: binary (power-of-two) scaling, np.round not int() truncation, explicit saturation. (fixed_to_decimal.py is an empty stub - the inverse, x / 2^F,

fixed_point_to_dec(x,_fixed,F,W): converts fixed point representations into decimal. Divides x_fixed by (2^(F-1) to return x_dec

# 5. Hardware Procedure (AD2 + WaveForms)

The single most failure-prone part of the workflow. Follow exactly.

## 5.1 Transmit Setup

- Run transmitter_main.py. Verify console prints frame shape (18, 80) and the metadata footer ends in ",18".

- WaveForms Wavegen: W1 (Channel 1): Custom -> import ad2_i_waveform.csv. W2 (Channel 2): Custom -> import ad2_q_waveform.csv.

- Both channels: Sample Rate 500 kHz, Run continuous / Repeat infinite, Amplitude 1 V.

- Channels dropdown: Synchronized. Without this the two AWGs free-run and I/Q are offset by an arbitrary number of samples (bug 7.8).

- Make sure Scope is set to 500k sample rate as well or else I/Q signal wont line up

- Start with Run All (top-left). NEVER the per-channel Run buttons - individually started channels begin at different times even in Synchronized mode.

## 5.2 Capture

- Scope Ch1 from W1 (I), Ch2 from W2 (Q). Scope channels are inherently simultaneous; the skew risk is purely on the generation side.

- 500 kS/s, at least 4096 samples (2-3 frame repetitions; 8192 typical).

- Export -> Recieved_waveform.csv.

## 5.3 The I/Q Alignment Check

Run this whenever BER is mysteriously 0.5:

ci = np.correlate(i_capture, i_waveform, 'valid')

cq = np.correlate(q_capture, q_waveform, 'valid')

bi, bq = np.argmax(np.abs(ci)), np.argmax(np.abs(cq))

# REQUIRED: bi == bq (skew 0), both correlation signs positive,

# both peaks strong, swap-test (i vs q_waveform) peaks ~10x weaker.

| Observation | Diagnosis |
| --- | --- |
| Skew much greater than 0 | AWG channels not started together -> Section 5.1 last step |
| Swap-test peaks strong | Scope/AWG cables crossed (Ch1 and Ch2 swapped) |
| Negative correlation sign | A channel inverted (cabling or amplitude sign) |
| Both peaks weak | Wrong waveform loaded on AD2, or stale capture |

# 6. Fixed-Point Budget

From the BER-vs-(W, F) sweep against the float64 golden chain:

- W = 16, F = 13 is the adopted format (Q2.13 plus sign).

- F = 14 leaves 1 integer bit -> saturation risk on PAPR peaks. F <= 12 raises the quantization noise floor visibly in BER.

- Quantize only at RTL module boundaries (TX output, RX input), never inside blocks that model vendor IP (the FFT) - those have their own internal scaling schedules decided at HDL time.

Open HDL-time decisions that interact with this budget: FFT per-stage scaling vs block floating point; storing 1/H_est once per frame to avoid dividers in the equalizer datapath; bit growth in the SC correlator accumulators (P and R need their own width analysis - |P|^2 doubles the bit width).

Overflow is when numbers are above the max threshold width for W=16 and F = 13 that leaves 1 sign bit and 2 integer bits which leaves a range of -4 to +4. My simulation is given a 0.4062 peak which gives a 9.85x headroom

After applying the Fixed point representation in Python Simulation, I recorded a BER of 0.0473. ~2% increase from decimal

# 7. HDL Implementation

## 7.1 Setup

## After fixed point conversion, data are converted into 2-complment to setup testbenches to validate HDL behavior

| File | Description |
| --- | --- |
| preamble_timing.txt | 64 complex samples of timing_preamble_gen(N) after fixed-point quantization |
| preamble_channel.txt | same for channel preamble |
| data_symbols.txt | the 16 × 64 QPSK data symbols, fixed-point |
| frame_freq.txt | the assembled 18 × 64 frequency-domain frame |
| frame_time.txt | after IFFT, the 18 × 64 time-domain blocks (W=16, F=13) |
| frame_with_cp.txt | after CP prepending, 18 × 80 |
| signal.txt | flattened, normalized, fixed-point — the 1440 samples that should come out of UART |

## File Format: one complex sample per line, two hex values separated by a space. For W=16 these are 4-character hex (two's complement):

1A2F E891

0C30 F2A4

...

# Fixed point to hex -> if positive convert integer to binary else convert to binary & 0xFFFF

Python Refrence

x — format as hexadecimal (lowercase). This is the conversion type. 5793 → 16a1.

4 — the minimum field width: pad the result to at least 4 characters.

0 — the padding character is zero (not space). So short values get leading zeros.

# Hex to fixed point mask = (1 << W) - 1 # W ones: W=16 -> 0xFFFF digits = (W + 3) // 4 # hex digits needed: 16 bits -> 4 digits return format(value & mask, f'0{digits}x')

## 7.2 ROM Preamble Design

## A read-only memory that stores frequency-domain OFDM samples.

## Interface:

## clk (input) — system clock

## addr (input, $clog2(DEPTH) bits) — address to read

## data_out (output, cplx_t) — sample at that address

## Parameters:

## DEPTH — number of entries (default 64)

## INIT_FILE — hex file with initial contents

## Behavior:

## On power-up / configuration, mem is loaded from INIT_FILE (one packed 32-bit entry per line, re in [31:16], im in [15:0]).

## On every rising edge of clk, data_out is updated to mem[addr]. This is a 1-cycle read latency.

## No reset port. data_out value during the first cycle after configuration is undefined; the consumer must not use it until after at least one rising edge with a valid address applied.

## mem contents are static after initial load.

## Out of scope: No arithmetic, no saturation, no write port.

## 7.3 IFFT Wrapper Design

## Wraps the Xilinx FFT IP xfft_0 to present a clean cplx_t streaming interface. Performs one-time configuration on startup (inverse FFT, scaling schedule [2 2 2] for ÷N). Converts between the project's {RE, IM} convention and the IP's {IM, RE} convention. Internally counts 64 input samples per block and generates tlast for the IP.

## Interface:

## clk (input) — system clock

## n_rst (input) — active-low async reset

## Input stream (slave):

## in_data (input cplx_t) — frequency-domain sample

## in_valid (input logic)

## in_ready (output logic)

## Output stream (master):

## out_data (output cplx_t) — time-domain sample

## out_valid (output logic)

## out_ready (input logic)

## out_last (output logic) — asserted on the 64th sample of each output block

## Behavior:

## After reset, the wrapper enters CONFIG state. It drives s_axis_config_tdata = 8'h54 and s_axis_config_tvalid = 1 on the IP's config port. When s_axis_config_tready = 1, the wrapper deasserts s_axis_config_tvalid and transitions to RUN.

## In CONFIG state, in_ready is held low — the wrapper rejects any input data until config completes.

## In RUN state:

## in_data is bit-swapped to the IP's TDATA format before forwarding (s_axis_data_tdata = {in_data.im, in_data.re}).

## s_axis_data_tvalid = in_valid; in_ready = s_axis_data_tready.

## An internal 6-bit counter in_count tracks input samples. s_axis_data_tlast is asserted on the 64th sample (when in_count == 63 and in_valid).

## On the output side:

## IP's m_axis_data_tdata is bit-swapped back to cplx_t (out_data = {re: m_axis_data_tdata[15:0], im: m_axis_data_tdata[31:16]}).

## out_valid = m_axis_data_tvalid; m_axis_data_tready = out_ready.

## out_last = m_axis_data_tlast. (Just forwarded from IP, since IP emits its own tlast on the 64th output sample.)

# 8. Bug Ledger (what failed, why, and the rule that prevents it)

Each of these cost real debugging time. The rule is the thing to carry forward.

| # | Bug | Symptom | Root cause | Rule |
| --- | --- | --- | --- | --- |
| 8.1 | Sync run on reshaped/FFT data | BER 0.5, garbage d_peak | Reshape assumes sample 0 is a frame boundary - circular | Sync lives in sample-space. Metric runs on the flat 1-D stream; reshape only after d_peak exists |
| 8.2 | CP_LEN = ceil(0.8e-6 x 500e3) = 1 | ISI even with perfect sync | Guard time shorter than one sample period | CP_LEN must exceed channel delay spread in samples; validate against the channel model |
| 8.3 | Constant 1+1j preamble | Flat sync metric (peak 1.3 vs median 0.84) | Impulse-train time signal; ~97% zero samples | Preambles use PN sequences, never constants |
| 8.4 | H_est divided by timing preamble | inf/nan on all odd bins | Timing preamble has zero odd bins | Channel estimation needs a full-spectrum preamble (or interpolation) |
| 8.5 | bits.txt = 2065 bits | BER 0.5, no crash | 17 data symbols generated, metadata claimed 18 | Payload = 2 x N x NUM_DATA exactly; compute NUM_BLOCKS from N_symbols.shape[0] |
| 8.6 | metric = metric[energy > thr] | Wrong d_peak, silent | Boolean filter shrinks array, breaks index-to-sample map | Gate by zeroing in place, never by filtering |
| 8.7 | Manual constellation snapping loop | Crash / row-vs-symbol confusion | data_eq[x] indexes rows of (16, 64), not symbols | Demapper already does nearest-quadrant; no pre-snapping. Flatten before symbol-level ops |
| 8.8 | I/Q skew 1039 samples | BER 0.5, blob constellation, I-I and Q-Q each correlate fine | AWG channels started independently | Synchronized mode AND Run All; verify with Section 5.3 |
| 8.9 | Stale capture / stale code | Any of the above, intermittently | TX changed without regenerate-reload-recapture | Any TX change: regenerate CSVs, reload AD2, recapture, then run RX. Metadata field count is the version marker |

Meta-pattern across 7.1 / 7.6 / 7.7: layer confusion - applying sample-level operations to block-level arrays or vice versa. When indexing anything, state to yourself whether the index is a sample, a block/row, or a subcarrier/column.

# 9. Extension Points (how to build on this)

## 9.1 Radar Path (next, in progress)

The frame already carries everything radar needs. Implementation plan:

# Y[k, n]: received freq-domain data symbols -> freq[2:].T  (64 subcarriers x 16 symbols)

# X[k, n]: KNOWN transmitted data symbols    -> rebuild from bits.txt via mapper

H_radar = Y / X                      # symbol division: strips data modulation

rd_map  = np.fft.ifft(               # 2D transform:

np.fft.fft(H_radar, axis=1),  # across symbols    -> Doppler axis

axis=0)                         # across subcarriers -> range axis

# |rd_map|^2 peaks = targets at (range bin, Doppler bin)

Key numbers at current parameters:

- Range resolution c / (2 x BW) = 300 m per bin (64 bins -> 19.2 km unambiguous). Fine for algorithm validation with synthetic delays; useless for real indoor targets - that requires more bandwidth (SDR front-end, Section 8.4).

- Doppler resolution 1 / (NUM_DATA x T_block) = 1 / (16 x 160 us) ~ 488 Hz per bin.

- Validation target: synthetic 2-target channel (distinct delay + Doppler), confirm two separated peaks in the map. This is the radar equivalent of "BER -> 0".

Keep compute_range_doppler(Y, X) -> rd_map as a pure function - it becomes the ML feature extractor later (Section 8.5) and the HDL block spec.

## 9.2 CFO Correction (closes the 2% BER gap)

The sync correlator already computes the CFO estimate for free:

eps = np.angle(P[d_peak]) / L        # phase per sample

r_corrected = r * np.exp(-1j * eps * np.arange(len(r)))

Apply before the frame slice. Diagnose first: if per-symbol BER grows across the frame, it is residual phase drift (CFO); if flat, it is noise-limited and CFO correction will not help much.

## 9.3 Recursive Schmidl-Cox (required before HDL)

The O(L)-per-sample loop is simulation-only. The streaming form is O(1) per sample:

P(d+1) = P(d) + conj(r[d+L]) * r[d+2L] - conj(r[d]) * r[d+L]

R(d+1) = R(d) + |r[d+2L]|^2            - |r[d+L]|^2

Two delay lines (length L and 2L) plus two accumulators. On FPGA, replace the division in M(d) with a threshold compare: detect |P|^2 > k x R^2 (one multiply, one compare). Pick k empirically in Python now (somewhere below the observed plateau height, well above the noise floor - current hardware numbers suggest k = 0.3 to 0.5).

## 9.4 Wideband RF Front-End

The AD2 caps the project at 500 kHz baseband. Next hardware step: SDR (HackRF ~20 MHz BW at 2.4/5.8 GHz, or PlutoSDR). 20 MHz gives 7.5 m range bins - real targets become visible. The Python chain is carrier-agnostic; only sample rate, N, and CP_LEN re-derivation are needed (CP must still exceed the new channel delay spread in the new sample units).

## 9.5 ML Hooks (far future, but shapes interfaces now)

- Classification operates on |rd_map| images (or range-Doppler-time stacks) - hence the pure-function requirement in Section 8.1.

- Data generation = the existing simulation chain in a loop with randomized target parameters plus labels. Build it as a generator, not a script.

- Keep detection (CFAR or learned) as a separate stage after rd_map so it can be swapped.

## 9.6 HDL Block Order (when the FPGA phase starts)

Smallest-first, each diffed against the Python golden model at W=16/F=13: mapper/demapper -> recursive SC correlator -> FFT (shared-resource design: comms needs 1 FFT per symbol, radar needs FFTs across the frame; at 500 kS/s one pipelined FFT serves both with large margin) -> channel estimator (compute 1/H once, multiply thereafter) -> equalizer -> symbol-division plus 2D transform.

# 10. Reproducibility Checklist

- bits.txt: exactly 2048 chars of 0/1, single line, no trailing whitespace.

- python transmitter_main.py -> frame (18, 80), metadata ends ",18".

- WaveForms: load both CSVs, Synchronized, 500 kHz, Repeat infinite, Run All.

- Scope: both channels, 500 kS/s, at least 4096 samples -> Recieved_waveform.csv.

- python receiver_communication.py -> expect metric_peak >= 0.7, median ~0.02, non-finite H bins: 0, BER <= 0.05.

- If BER ~0.5 -> run the Section 5.3 I/Q alignment check before touching any code.

# 11. Glossary

| Term | Meaning here |
| --- | --- |
| d / d_peak | Candidate / detected frame-start sample index in the raw stream |
| L | Half-symbol length, N//2 = 32 |
| P(d), R(d), M(d) | SC half-correlation, second-half energy, normalized timing metric |
| plateau | Flat top of M(d), width ~CP_LEN; any point on it is a valid frame start |
| symbol division | Y/X - removing known data modulation to expose the channel (radar step) |
| sample-space | The flat 1-D received stream, before any block structure exists |
| golden model | The float64 Python chain; every fixed-point/HDL block is diffed against it |

End of documentation. The Python implementation in the smanthat/OFDM repository is canonical; this document supplements it.